Group 47

Choong Jin Ng | 301226977

Ryan Kiew Ruelt Yean | 301290779

Sachin Sac | XXXXX7150

ENSC 350 Final project part 1 report

Table of Contents

Introduction	2

LogicUnit	2

Overview	2

Functional Behaviour	3

Circuit Synthesis	5

Timing Simulations	7

ArithUnit	8

Overview	8

Functional Behaviour	11

Circuit Synthesis	12

Timing Simulations	15

Conclusion	16

Appendix	17

Appendix A: Logic Gates	17

Table of Tables	19

Table of Figures	19

# Introduction

The objective of part 1 of this project is to design, synthesize, and test an Arithmetic Logic Unit (ALU) – the first step in assembling a processor. The ALU is responsible for performing arithmetic and bitwise operations on integer binary numbers, and is an essential building block of many other types of computing circuits.

This ALU design is separated into two parts, the Logic Unit and the Arithmetic Unit; the Logic Unit appropriately performs logical bitwise operations on two 64-bit input signals while the Arithmetic Unit is responsible for producing the arithmetic result of two 64-bit input signals, depending on certain context variables.

After designing the respective units, the circuits are synthesized using Quartus Prime’s RTL netlist viewer. The circuits will be synthesized for a Cyclone IV FPGA. We will then be using ModelSim to perform functional and timing simulations, which will be compared to the provided testbench values to verify that the units are working as intended.

# LogicUnit

## Overview

The LogicUnit is responsible for selecting and operating Logic Bitwise operations of two 64-bit input signals, A and B. This design incorporates the following operations:

- Pass the signal of B

- The result of A xor B

- The result of A or B

- The result of A and B

These initial logical operations are computed immediately, with the results passed along by a multiplexer as signal Y, depending on the signal LogicFn. The block diagram of the LogicUnit is represented in Figure 1 and the truth table of the LogicUnit is indicated in Table 1. The VHDL representation is given in Figure 2.

Figure 1: Block Diagram of LogicUnit Circuit

| LogicFn Signal | Operation (Signal Y) |
| --- | --- |
| 0 0 | B |
| 0 1 | A xor B |
| 1 0 | A or B |
| 1 1 | A and B |

Table 1: Truth Table of LogicUnit

Figure 2: VHDL Interface of LogicUnit

## Functional Behaviour

The functional behaviour of LogicUnit can be demonstrated in three example diagrams Figure 3, Figure 4 and Figure 5. In Figure 3, the different LogicFn signals are demonstrated.

Figure 3: Functional Simulation for LogicUnit from t=0 to t=70ns

Figure 4: Functional Simulation for LogicUnit from measurement #458 to measurement #464

Figure 5: Functional Simulation for LogicUnit from measurement #770 to measurement #772

## Circuit Synthesis

As indicated, the three logical diagrams are XorGate, AndGate, and OrGate. The VHDL interface for the three block diagrams and synthesised circuits are listed in Appendix A: Logic Gates.

Figure 6: Synthesised Circuit of LogicUnit

## Timing Simulations

Figure 7: Timing Simulation for LogicUnit of Measurement #1

Figure 8: Timing Simulation for LogicUnit from measurement #400 to measurement #416

Figure 9: Timing Simulation for LogicUnit of Measurement #772

# ArithUnit

## Overview

The ArithUnit is responsible for producing the appropriate arithmetic result depending on the context. The input context variables here are:

- A, B :  64-bit input signals to be used in the adder

- AddnSub : determines whether the operation carried out is an add or a subtract

- NotA : to be used later for retrieval of instructions

- ExtWord : determines whether or not to sign extend the value

The output variables are:

- Y : result of the arithmetic operation

- Cout : outgoing carry of the result

- Ovfl : signifies an overflow in the result

- Zero : signifies if A is equal to B

- AltB, AltBu : signed and unsigned flags that indicate whether A is less than B

The ArithUnit was designed to handle both 64 and 32-bit numbers through the ExtWord flag. It currently does not support any other operations other than addition and subtraction. If the operation performed is a subtraction, it simply negates the value in B and feeds it into the adder. The output signals are designed with future function implementations in mind. For example, AltB and AltBu may seem insignificant, but will eventually be used to implement branching instructions in the future.

The block diagram of the ArithUnit is represented in Figure 10. The VHDL interface of the ArithUnit is given in Figure 11.

Figure 10: Block diagram of the ArithUnit

Figure 11: VHDL interface of the ArithUnit

The adder we used to is a simple ripple adder. The adder performs the addition bit-by-bit and propagates any carry that exists. Other than the result of the operation, it also returns the outgoing carry value, Cout, and the Overflow flag, Ovfl. Cout is simply the final carry value of the carry array, and Overflow is computed as the XOR of the last and second-to-last carry values of the carry array. These values are used in the ArithUnit to compute AltB and AltBu.

The block diagram of the Adder is represented in Figure 12. The VHDL interface of the Adder is given in Figure 13.

Figure 12: Block diagram of the Adder

Figure 13: VHDL interface of the Adder

## Functional Behaviour

Figure 14: Functional Simulation of the Arithmetic Unit

We can confirm that the ArithUnit is functioning as intended by observing that the Unit’s Y values are the same as the testbench – TbY. This figure also encapsulates all the different combinations of the context variables and its resulting output. For example, as annotated on the figure, we have a test case where the arithmetic operation is an ADD with no sign extension, that produces a Cout and no Overflow. We also have another test case that showcases SUBTRACT with sign extension, that similarly produces a Cout with no Overflow, but this time the AltB flag is also set.

Figure 15: RTL synthesized circuit of the ArithUnit

Circuit Synthesis

Figure 16: Panned-out view of the RTL synthesized circuit of the ArithUnit

Figures 15 and 16 represent the RTL synthesized circuit of the ArithUnit. The circuit is organized so that every input bit can be observed from the figure. The functional logic portion of the unit are mostly performed in parallel, and only take up two layers of delay before producing the final result. The ripple adder used in this circuit is represented in green, and the synthesized circuit diagram of it is displayed in Figure 17. Since a basic ripple adder was used, the resulting propagation delay is quite significant. We could reduce this delay by implementing a carry-skip or Brent Kung adder instead. The output of the Adder is then used in the ArithUnit in the process of obtaining the final output value Y.

Figure 17: RTL synthesized circuit diagram of Adder

Figure 18: TIming Simulation for the ArithUnit

Timing Simulations

The timing of the ArithUnit can be verified by observing that the results are obtained well within the allocated time period. For measurement #1, the total propagation delay is 18.7 ns, with 8.3 ns to spare. The ripple adder took 7.3 ns to compute the appropriate result – roughly 40% of the entire time taken. If we were to improve this circuit in the future, improving the implementation of the adder will significantly improve the circuit’s efficiency.

# Conclusion

In this report, we outline two units for computational purposes and its performance. The Logic Unit determines whether bit-wise logical operations are performed while the Arithmetic Unit adds two 64-bit inputs together. These two combined will form the Arithmetic Logic Unit, the first steps to a general-purpose processor. Their propagation delay is respectively X and Y.

# Appendix

## Appendix A: Logic Gates

Inside the LogicUnit, the logic gates are represented by three entities, AndGate, OrGate and XorGate. They are use the operators and, or and xor which are available in the IEEE standardised library. The logic gates entity interfaces and implementation for AndGate, OrGate and XorGate is represented in Figure 19, Figure 20 and Figure 21 respectively.

Figure 19: VHDL Interface and Implementation of AndGate

Figure 20: VHDL Interface and Implementation of OrGate

Figure 21: VHDL Interface and Implementation of XorGate

The synthesised circuits are represented in Figure 22.

Figure 22: Synthesised Circuits of AndGates, OrGates and XorGates respectively

# Table of Tables

Table 1: Truth Table of LogicUnit	3

# Table of Figures

Figure 1: Block Diagram of LogicUnit Circuit	3

Figure 2: VHDL Interface of LogicUnit	3

Figure 3: Functional Simulation for LogicUnit from t=0 to t=70ns	4

Figure 4: Functional Simulation for LogicUnit from measurement #458 to measurement #464	4

Figure 5: Functional Simulation for LogicUnit from measurement #770 to measurement #772	5

Figure 6: Synthesised Circuit of LogicUnit	6

Figure 7: Timing Simulation for LogicUnit of Measurement #1	7

Figure 8: Timing Simulation for LogicUnit from measurement #400 to measurement #416	7

Figure 9: Timing Simulation for LogicUnit of Measurement #772	8

Figure 10: Block diagram of the ArithUnit	9

Figure 11: VHDL interface of the ArithUnit	9

Figure 12: Block diagram of the Adder	10

Figure 13: VHDL interface of the Adder	10

Figure 14: Functional Simulation of the Arithmetic Unit	11

Figure 15: RTL synthesized circuit of the ArithUnit	12

Figure 16: Panned-out view of the RTL synthesized circuit of the ArithUnit	13

Figure 17: RTL synthesized circuit diagram of Adder	14

Figure 18: TIming Simulation for the ArithUnit	15

Figure 19: VHDL Interface and Implementation of AndGate	17

Figure 20: VHDL Interface and Implementation of OrGate	17

Figure 21: VHDL Interface and Implementation of XorGate	18

Figure 22: Synthesised Circuits of AndGates, OrGates and XorGates respectively	18